Summary on "a4"

- CC2091: Was assembled with rotated collar packs

- CC2090: Was collared being introduced in the collaring press reversed (tilted along the vertical axis).

- CC2098, CC2119, CC2121, CC2123 and CC2124 were measured before collaring (after precollaring).

- CM2129 was measured tilted by 180 degrees around longitudinal axis.

- CC2139, CC2140, CC2141, CC2142, CC2145 and CC2146 were (or will be) precollared with smaller pins (10mm instead of 11mm).

- CC2092: after decollaring because of block 6 movement suspects 1300mm of packs were deplaced from splice region and the whole packs shifted by this length.

- 2137: Collared coil rotated by 180 degrees in cold mass yokes.

- From collared coil 2091 to collared coil 2100: the poles were assembled upper/lower in the opposite way by respect to the standard (odd/even numbers).