| – |
| --- |
| Nonconformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name:Monique Dupont | 6. Date: 2003-09-08 |
| 2. Contractor/Supplier: Ansaldo | 7. Part description:LHC cryodipole |
| 3. Contract No: | 8. Qty:1 |
| 4. Project Engineer: dipcoor | 9. Dwg No: |
| 5. Quality Manager: |  |

| 10. Found during what activity: |  |
| --- | --- |
| X Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

11. Description of nonconformity (use continuation page if necessary)

End cover orientation around X out of tolerance on lyre side Delta Y of V1 cold bore tube extremity on connection side out of tolerance Radial and vertical positions of line N on connection side out of tolerance

12. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 13. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 14. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

15. Description of proposed action (use continuation page if necessary)

| APPROVAL OF NON CRITICAL NONCONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Engineer: | Date: |

| APPROVAL OF CRITICAL NONCONFORMITIES |  |  |
| --- | --- | --- |
| 17 | Project Management: | Date: |

| CLOSURE OF THE NONCONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical nonconformities Quality Manager or Project Engineer | For critical nonconformities Project Engineer |
| 18 | Name: Date: | Name: Date: |

NONCONFORMITY CONTINUATION PAGE

- End covers data :

| Lyre side |  | D9-D10-D11 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | X | Y | Z |  |  |
|  | D9 | 0.07 | 0.44 | -0.16 |  |  |
|  | D10 | 0.01 | 0.54 | 0.37 |  |  |
|  | D11 | 0.32 | 0.24 | 0.14 |  |  |
|  |  | Center of D9-D10-D11 |  |  | Tol(X) | Tol(Z) |
|  | C | 0.04 | 0.49 | 0.11 | 0.9 | 0.9 |
|  |  | Angle around X axis (mrad) |  |  | Tol(ec) |  |
|  | ec | 1.03 |  |  | 0.75 |  |
|  |  | Angle around Y axis (mrad) |  |  |  |  |
|  | ec | 1.07 |  |  |  |  |
|  |  | Angle around Z axis (mrad) |  |  |  |  |
|  | ec | -0.21 |  |  | 0.75 |  |

- Cold bore tube extremities :

|  | COLD BORE TUBE EXTREMITIES |  |  |  |  |  |  | Tol(Y) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connection side | V1 FLANGE |  |  |  | V2 FLANGE |  |  | - 0.7/ 0.3 |
|  | Center (Outer diameter) |  |  |  | Center (Outer diameter) |  |  |  |
|  | X | Y | Z | Tol(Y) | X | Y | Z |  |
|  | 0.16 | 0.33 | 0.04 | - 0.7/ 0.3 | 0.17 | 0.17 | 0.30 |  |
|  | R |  | Tol(R) |  | R |  | Tol(R) |  |
|  | 0.16 |  | ± 1 |  | 0.35 |  | ± 1 |  |
|  | Angle/V1 (mrad) |  | Tol(V1c) |  | Angle/V2 (mrad) |  | Tol(V2c) |  |
|  | 2.1 |  | ±4.5 |  | 2.4 |  | ±4.5 |  |

- Cartography connection side :

| Point | X (mm) |  |  |  | Z (mm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Measured | Theo | Delta | Tolerance | Measured | Theo | Delta | Tolerance |
| Centre_E | -269.30 | -270.00 | 0.70 | +/- 2 | -288.07 | -290.00 | 1.93 | +/- 2 |
| Centre_M1 | -139.98 | -140.00 | 0.02 | +/- 1.15 | 145.72 | 145.00 | 0.72 | +/- 1.15 |
| Centre_M2 | 140.01 | 140.00 | 0.01 | +/- 1.15 | 145.50 | 145.00 | 0.50 | +/- 1.15 |
| Centre_M3 | -144.73 | -145.00 | 0.27 | +/- 1.15 | -144.45 | -145.00 | 0.55 | +/- 1.15 |
| Centre_N | 315.13 | 310.00 | 5.13 | +/- 2 | -67.45 | -70.00 | 2.55 | +/- 2 |
| Centre_V1 | -97.10 | -97.26 | 0.16 | +/- 1 | 0.04 | 0.00 | 0.04 | +/- 1 |
| centre_V2 | 97.43 | 97.26 | 0.17 | +/- 1 | 0.30 | 0.00 | 0.30 | +/- 1 |
| Centre_W (cryostat) | 1.96 | 0.80 | 1.16 | +/- 2 | -78.98 | -80.00 | 1.02 | +/- 2 |
| Centre_X | -0.45 | 0.00 | -0.45 | +/- 1.15 | 180.33 | 180.00 | 0.33 | +/- 1.15 |

INSTRUCTIONS FOR COMPLETING THE NONCONFORMITY REPORT

| 1. Originator | Name of the person who identifies the nonconformity |
| --- | --- |
| 2. Contractor/Supplier | Organisation where the nonconformity is detected |
| 3. Contract No | CERN's contract or order No |
| 4. Project Engineer | Name of the CERN or Institute engineer in charge of the contract |
| 5. Quality Manager | Name of the person responsible for quality control |
| 6. Date | Date when the nonconformity is identified |
| 7. Part description | Name of the part such as it appears on drawing or contract or order |
| 8. Qty | Number of parts or lots affected |
| 9. Dwg No | Part drawing number and revision index |
| 10. Found during what activity | Tick the appropriate box. If ticking Other explain the circumstances |
| 11. Description of the nonconformity | Describe the problem, identify the requirements that are not met, give references to specifications, procedures etc. If possible describe the possible causes of the nonconformity, such as inadequate procedure, wrong test set-up and so on. |
| 12. Action taken to prevent misuse | Describe what steps have been taken to ensure that the item is segregated from the normal production while the nonconformity remains unresolved. |
| 13. Importance | P.E. to decide if the nonconformity is critical or not and tick appropriate box |
| 14. Disposition | P.E. to decide on disposition, tick appropriate box and outline the details of the proposed actions. |
| 15. Corrective/preventive action | P.E. to decide what action should be taken with the design, the manufacturing process, the testing procedure or any other circumstance to prevent the reoccurrence of the problem. |
| 16. Approval of non critical nonconformities | Complete with the name of the Project Engineer and the date of approval. |
| 17. Approval of critical nonconformities | Complete with the name of the Project Manager, the name of the approval list if appropriate, and the date of approval. |
| 18. Closure of the nonconformity | For a non critical NC, complete with the name of the Quality Manager and the date of the verification. For a critical NC, complete with the name of the CERN Project Engineer and the date of the verification. |

Note that points 16, 17 and 18 may be left blank for all nonconformities that are tracked using the EDMS system as described in chapter 3 of document LHC-PM-QA-611.00 “Management of Nonconformities”