| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: B.Caserza | 5. Date: 19/05/2003 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: HCMBBRA001-02000021 |
| 3. Contract No: F302/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary | 8. Dwg No: LHCMBB_A0002/F (683RM08451) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection | Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

Final geometrical measurements performed on cold mass nr. 2021 showed out-of-tolerance about: - cold mass total length - twist between V1 & V2 tubes See next page for details.

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) - 1. The NC regarding the length of the cold mass: - The cold mass total length at 20C is 15160.8 mm instead of the maximum allowable length of 15160.4mm.We propose to make a repair in the following way: respecting the tolerance of 64 +/-0.55 between the flanges and the end covers (on connection and lyra side) reduce the total distance between flange and flange to the maximum allowable: 15160.4 + 2* 64.5 =15289.2mm. The correction should be made at Ansaldo before delivery of the cold mass at CERN. The position of the M and X lines will remain out of tolerance on each side of 0.3mm according to the cold mass length. - 2. The NC regarding the twist: - Is acceptable as the new tolerances allow to accept mechanical local twist up to +/-6mrad. |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary)

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: Marta Bajko | Date: 5/21/03 |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: Cryodipole coordination | Date: 5/28/03 |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: Date: | Name: P. Gagliardi Date: 29/05/2003 |

X

A

D

B

C

V1 line flange

end-cover ref. plane

30/05/2003

|  | CONN. | SIDE |  |  | LYRE | SIDE |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pos. | X before adjust. | X after adjust. | X(CS) |  | X before adjust. | X after adjust. | X(LS) |
| A | 0 | -0.30 | -0.30 |  | 0 | -0.30 | -0.30 |
| B | +0.14 | -0.18 | -0.32 |  | 0 | -0.32 | -0.32 |
| C | +0.26 | -0.08 | -0.34 |  | +0.16 | -0.18 | -0.34 |
| D | +0.09 | -0.24 | -0.33 |  | +0.06 | -0.26 | -0.32 |

Results of geometrical measurements on V1 flanges (CS and LS) before and after mechanical adjustment.

B. Caserza - F. Terzi