| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: F. Terzi | 5. Date: 02/10/2003 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: HCMBARA001-02000029 |
| 3. Contract No: F302/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary | 8. Dwg No: LHCMBB_A0002/F (683RM08451) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

Final electrical measurements performed on CM 2029 showed a wrong value about corrector magnet MCO Apt1 – Bus 11/EE841. Measured value=3293.69 mohm Nominal value > 6500 mohm. See next page for details.

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary)

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: | Date: |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: | Date: |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: Date: | Name: Date: |

| R[dc] of the Spools |  |  | ITP 25c |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I = | 1.00 | A | Apt 1 | [ext] |  | Apt 2 | [int] |
|  | Connection Side |  | [m |  |  | [m |  |
|  | MCO |  | 3293.69 | EE841/Aux.2 |  | 3283.80 | EE941/Aux.12 |
|  |  | Nr. MC0 | MA-I-0210 |  |  | MA-I-0211 | #RIF! |
|  | MCD |  | -4317.86 | EE851/Aux.4 |  | -2110.00 | EE951/Aux.14 |
|  |  | Nr. MCD | MA-I-0210 |  |  | MA-I-0211 |  |
|  | Lyra Side |  |  |  |  |  |  |
|  | MCS |  | -2312.94 | EE831/Aux.10 |  | -104.96 | EE931/Aux.20 |
|  |  | Nr. MCS | MA-I-0155 |  |  | MA-I-0243 |  |