| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: F. Terzi | 5. Date: 16/01/2004 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: Press for E-mod. meas. on coil heads |
| 3. Contract No: F462/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary | 8. Dwg No: 99444544701 (Supplier: REJLERS) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

(see next page)

11. Action taken to prevent misuse (use continuation page if necessary)

A new procedure for shim calculation has been developed in agreement with CERN (ASG ref. doc. nr. 420RM9846) Further investigation will be done in collaboration with CERN.

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary)

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: | Date: |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: | Date: |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: Date: | Name: Date: |

IDENTIFICATION

10. Description of non conformity (continuation page)

A dimensional check on REJLERS-CERN press, for E-modulus measurements on coil heads, has been performed by ASG in collaboration with CERN people.

This check showed that the dimension of the cavity was lower than the nominal one, moreover a deformation of the heigth of two sizing bars, from the nominal value 22.0 mm to 21.76 mm (bar nr.1) and 21.48 mm (bar nr.2) – (average values), was detected.

Table 2. Dimension of the sizing bar as measured on 28th of November 2003

(extract from AT-MAS/MB 7265)

See CERN Visit Report nr. AT-MAS/MB 7265 dated 18-21/12/2003 for further details.

The failure of the press caused wrong measurements of E-modulus of coil heads regarding different collared coils.

As a consequence, the shim calculation gave wrong results and wrong shims on the heads were installed on several collared coils.

Some of these collared coils were assembled in cold masses.