| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: B.Caserza – F. Terzi | 5. Date: 13/02/2004 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: HCMBALA001-02000072 |
| 3. Contract No: 462/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary, M. Bajko | 8. Dwg No: LHCMBB_A0002/F (683RM08451) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

Final geometrical measurements performed on c.m. 2072 showed out-of-tolerance about the positioning of the cold feet pads after welding. Measured values of distance between feet and midplane are: - 293.169 mm on CONNECTION SIDE (nom. value = 294.00 ±0.6 mm.) - 293.247 mm on CENTRAL POSITION - 293.093 mm on LYRE SIDE See next page for more details.

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) - To adjust the before mentioned dimension, a shim of a 0.5mm thickness was fixed on the 3 cold feet pads. |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary) The action proposed is acceptable nevertheless there are two points on to be mentioned: - the cold feet pads by fabrication should be such that this errors theoretically are avoided; therefore we ask Ansaldo to check with attention the dimensions of the cold feet pads delivered by your suppliers.. In this particular case we aske Ansaldo to present the CC of the pads assembled on this cold mass - the longitudinal position of the cold foot on the connection side is on the limit of the tolerance. At this stage of the assembly a better positioning of the pad could and should be made.

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: Marta Bajko | Date: 13.02.2004 |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: | Date: |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: F. Terzi Date: 13/02/2004 | Name: Date: |

10. Description of non conformity (continuation page)

Please find here below the results of geometrical test regarding feet positioning. (ref. to Excel data file ITP20-HCMBA-2072_compare_3.xls).

F. Terzi