| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: F.Terzi | 5. Date: 08/06/2004 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: 02P00403 |
| 3. Contract No: F462/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary, M. Bajko | 8. Dwg No: LHCMB__A0001/G (620RM08234/L) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection | Final inspection |
| In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

After collaring of CC 2101, electrical test showed a turn to turn short circuit on pole 02P00403 (pos. D1-lower).

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) - The collared coils assembly cannot be used as is. - The pole shall be repaired if possible. If repairing is not possible, then the pole shall be rejected. - CERN suggested repeating the warm magnetic measurements (which, in case of sustained short, give key information for localizing better the short). - These measurements have been carried out on the c.c. assembly being turned upside down and consequently the short has been seen in D2-Upper. |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary)

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: F. Savary | Date: June 14th, 2004 |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: | Date: |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: F. Terzi Date: 16/09/2004 | Name: Date: |

CLOSURE OF THE NON CONFORMITY

Collared coils CC 2101 have been dismantled.

Electrical check has been performed on pole 02P00403 in order to find out the position of the short circuit.

An interturn short circuit was found between 6th and 7th turn on I.L. 02I00406.

The layer has been repaired and electrical test performed with good result.

F. Terzi

1/2 – Before repairing

2/2 – After repairing