| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: F. Terzi | 5. Date: 26/11/2004 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: Bus-bar B-360 |
| 3. Contract No: F462/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary, M. Bajko | 8. Dwg No: HCDCBHA055 |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

During electrical check of bus-bar nr. B-360 (id. nr. HCDCBHA056-BI000360) just before longitudinal welding of CM 2137, a low insulation resistance has been found. Cause of this problem seems to be high humidity / water, get during transport of bus-bars. Same problem we found on bus bars regarding set nr. from A-336 to A-345 and from B-351 to B-360 (ref. to NC CERN 268)

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) ASG proceeded with dismantling the cold mass active part (not already welded) and replacing the affected bus bars. |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary)

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: F. Savary | Date: March, 8th 2005 |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |  |
| --- | --- | --- | --- |
| 16 | Project Management: |  | Date: |
| CLOSURE OF THE NON CONFORMITY |  |  |  |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |  |
| 17 | Name: F. Terzi Date: 16/03/2005 | Name: Date: |  |