|  | NON CONFORMITY REPORT | Nr. NC CERN 47 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC020405A (ANSALDO NC id. num.) Date: 05/04/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION HCMBB_A001-02000002 Collared coils nr. M002 |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| With reference to NC CERN 42, after dismount of the collars, on 21-22-25/03/02, with CERN personnel witness, |  |  |  |  |  |  |  |  |
| the following damages on QHs have been detected: |  |  |  |  |  |  |  |  |
| - QHs n.YT211 & YT212 (N.C.S. - coil end) are burnt very close to the electrical connection (“omega” shape piece) to the wires |  |  |  |  |  |  |  |  |
| 2) QHs n. YT111 (N.C.S. - coil end) are burnt very close to the electrical connection (“omega” shape piece) to the wires |  |  |  |  |  |  |  |  |
| These damages seem to be happened at CERN during electrical test at RT and cryogenic temperature |  |  |  |  |  |  |  |  |
| RESOLUTION |  |  |  |  |  |  |  |  |
| Waiting for CERN proposal. |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 05/04/2002 |  | 05/04/2002 |  |  |  |  | 05/04/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED |  |  NOT ACCEPTED |  |
| REMARKS |  |  |  |
| - No proposal from Ansaldo that is in fact waiting recommendations from CERN |  |  |  |
| - During a visit at Ansaldo premises on the 11th of July 2002, we (CERN representatives and Ansaldo staff) could see that there are also broken end spacers. CERN suggested Ansaldo trying to replace the broken end spacers before proceeding to re-assembly. This is in view of saving the coils. This subject will be reviewed after Ansaldo has tried to do this. |  |  |  |
| - As usual when a coils’ assembly has to be re-assembled and re-collared, the following components shall be replaced: - Ground insulation; - Quench-heaters; - Coils’ protection sheets and shim retainers; - Cold bore tubes (because they were cut during cold mass finishing). |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date 07.07.03 | Signature Frederic Savary |  |  |
| REMARKS |  |  |  |
| Ansaldo changed the end spacers on outer layer E007 on pole P008. Due to difficulties in doing this operation, Ansaldo decided to leave the end spacers of the other layers in place. All the required parts such as quench heaters, cold bore tubes, ground insulation, coil protection sheets and shim retainers were replaced |  |  |  |

MOD NC-CERN_2