|  | NON CONFORMITY REPORT | Nr. NC CERN 49 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC 020429A (ANSALDO NC id. num.) Date: 29/04/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION Inner layer I051 |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| After curing, inner layer nr. I051, has some turns detached along 200 mm about from end spacer, on both ends (CS and NCS) |  |  |  |  |  |  |  |  |
| This defect has been already showed to CERN people during visit at Ansaldo on 29/04/02. |  |  |  |  |  |  |  |  |
| RESOLUTION |  |  |  |  |  |  |  |  |
| We propose to restore the gluing between detached turns (both side), using epoxy resin at room temperature with layer |  |  |  |  |  |  |  |  |
| under pressure: this process will be done during the assembling of the inner and outer layer (pole). |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 29/04/2002 |  | 29/04/2002 |  |  |  |  | 29/04/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED |  |  NOT ACCEPTED |  |
| REMARKS |  |  |  |
| - Repairs to be carried under Ansaldo responsibility |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date | Signature |  |  |
| REMARKS |  |  |  |

MOD NC-CERN_2