|  | NON CONFORMITY REPORT | Nr. NC CERN 51 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC 020503B (ANSALDO NC id. num.) Date: 03/05/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION Cold mass HCMBA_A001-02000001 (c.c. M005) |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| - During electrical test on QHs of c.c. M005, (DIQH tests performed by CERN personnel on 02/05/02, according to tech. procedure nr. LHC-DHQ-TP-0001.00 rev.1.1), the QH nr.YT211 broke (open circuit) at 450 V discharge test (I=20A; E=715J). After discharge, no short circuits have been detected from QH (YT211) to ground or to coils. Remarks: QH resistance before discharge was R=20.8 Ohm. - Longitudinal welds of shrinking cylinders, performed by CERN and after repairing by CERN, are not acceptable: see AJC-CERN report dated 08/01/02 regarding nr.11 repairing on “TE” side. - After Ansaldo repairing of TE side longitudinal weld, some defects still remain and weld is not acceptable (see SIGE-ANSALDO report nr.02052-01-mt) |  |  |  |  |  |  |  |  |
| RESOLUTION |  |  |  |  |  |  |  |  |
| According to decision by CERN, this cold mass must be dismounted (cutting of cylinder, dismount of yokes, de-collaring of c.c.) in order to understand the QHs problem. |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 03/05/2002 |  | 03/05/2002 |  |  |  |  | 03/05/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED |  |  NOT ACCEPTED |  |
| REMARKS |  |  |  |
| - No proposal from Ansaldo to sort out the problem; - The assembly can not be accepted in such conditions. Therefore, it has to be dismounted to replace the quench heaters. |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date | Signature |  |  |
| REMARKS |  |  |  |

MOD NC-CERN_2