|  | NON CONFORMITY REPORT | Nr. NC CERN 59 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC 020703A (ANSALDO NC id. num.) Date: 03/07/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION Quench Heaters nr.4864 & 4779 (ref. pos. YT112 on c.c. M013) |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| After impulse test on collared coils nr.M013, performed at 60 A by CERN personnel, the resistance of QHs nr.4864 and 4779 (ref. pos. Y112) changed from 20.75  to 23.60 . Besides, as stated by CERN personnel, an anomalous behaviour of the current occurred during the electrical test. The measured insulating resistance value from QH to pole is acceptable: R=1.19 G (ref. value R>1.00 G). |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 03/07/2002 |  | 03/07/2002 |  |  |  |  | 03/07/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED |  |  NOT ACCEPTED, by F.Savary on the 18th of September 2002 |  |
| REMARKS |  |  |  |
| The collared coils have to be dismounted, the quench heaters replaced and the whole be re-collared (already done following discussions held in due time). |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date | Signature |  |  |
| REMARKS |  |  |  |

MOD NC-CERN_2