|  | NON CONFORMITY REPORT | Nr. NC CERN 64 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC 020719A (ANSALDO NC id. num.) Date: 19/07/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION Collared coils M011: QH nr. YT211 |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| Electrical test on c.c. M011, after collaring, showed QH nr. YT211 (D2–Upper pole–Left Side) interrupted. |  |  |  |  |  |  |  |  |
| We ask CERN how to proceed. |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 19/07/2002 |  | 22/07/2002 |  |  |  |  | 22/07/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED |  |  NOT ACCEPTED, by F.Savary on the 18th of September 2002 |  |
| REMARKS |  |  |  |
| Following an inspection of the damaged quench heater (ASC has removed several collar packs at the end to allow inspection of the critical zone), we decided to de-collar this c.c. assembly M011 to replace the damaged quench heater (omega cut at the junction with the quench heater strip). |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date | Signature |  |  |
| REMARKS |  |  |  |

MOD NC-CERN_2