|  | NON CONFORMITY REPORT | Nr. NC CERN 66 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC 020729A (ANSALDO NC id. num.) Date: 29/07/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION Collared coils M011: QH nr. YT211 |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| With reference to NC CERN 64, after de-collaring of coils on CS, we found the QH strip broken just on the connection to omega piece. Possible causes of this damage could be: 1) a cutting effect due to a no support surface under the QH strip end; 2) a too much high stress value during collaring on coil ends This damage has been already showed to CERN people (Mr. Savary and Mr. Musso) during their visit at Ansaldo. |  |  |  |  |  |  |  |  |
| Waiting for CERN feedback on this problem |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 29/07/2002 |  | 29/07/2002 |  |  |  |  | 02/08/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED |  |  NOT ACCEPTED, by F.Savary on the 18th of September 2002 |  |
| REMARKS |  |  |  |
| According to CERN e–mail dated 26/07/2002 from Mr.Vlogaert, “the collared coil shall be de-collared and the ground insulation, quench heaters and coil protection sheets shall be replaced by new ones”. |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date | Signature |  |  |
| REMARKS |  |  |  |

MOD NC-CERN_2