| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: B.Caserza | 5. Date: 22/11/2002 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: HCMBB_A001-02000006 |
| 3. Contract No: F302/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary | 8. Dwg No: LHCMBB_A0002/F (683RM08451) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

During electrical test performed before welding of end covers, open circuit of temperature sensor TT821 has been detected.

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  Non critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. |  Use-as-is |  Repair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) The electrical circuit of the affected temperature sensor shall be left in place but the wires shall be cut to a reasonable length and their extremity properly insulated. A new temperature sensor shall be put in place, fixed to the end plate. CERN staff will give recommendations on where to put the sensor and how to fix it at the occasion of a next visit. |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary) CERN suspects the welding operations to induce high currents in the instrumentation circuits. In order to minimize such risk, CERN recommends Ansaldo to follow strictly the “Installation Procedure” entitled “Pre-series MBA – MBB Cold Mass Instrumentation”, Rev.3 dated September 2002 (doc. Ref. LHC-MMS-GB/5837) by G.Brun.

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: F.Savary | Date: 28th January 2003 |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: | Date: |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: F. Terzi Date: 28/01/03 | Name: Date: |

NON CONFORMITY CONTINUATION PAGE

The new temperature sensor as installed on the end plate – “After repair situation”