| – |
| --- |
| Non conformity Report |  |
| IDENTIFICATION |  |
| 1. Originator’s Name: B.Caserza | 5. Date: 27/03/2003 |
| 2. Contractor/Supplier: Ansaldo Superconduttori S.p.A. | 6. Part description: HCMBBRA001-02000012 |
| 3. Contract No: F302/LHC/LHC | 7. Qty: 1 |
| 4. Project Engineer: F. Savary | 8. Dwg No: LHCMBB_A0002/F (683RM08451) |

| 9. Found during what activity: |  |
| --- | --- |
|  Incoming inspection |  Final inspection |
|  In-process inspection |  Other: |

10. Description of non conformity (use continuation page if necessary)

Final geometrical measurements performed on c.m. 2012 showed out-of-tolerance about the positioning of the cold feet pads after welding: measured values of distance between feet and midplane is 293.25 mm on CS and 293.50 mm on CENTER (nom. value = 294.00 ±0.5 mm.) See next page for more details.

11. Action taken to prevent misuse (use continuation page if necessary)

| IMPORTANCE |  |  |
| --- | --- | --- |
| 12. |  xNon critical |  Critical |

| DISPOSITION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13. | xUse-as-is |  xRepair |  Reject |  Rework |  Return to supplier |
| Description of proposed action (use continuation page if necessary) - Use a shim of 0.5mm on the 3 cold feet pads in order to correct the distance between the cold feet pads and the mid plane of the cold mass. |  |  |  |  |  |

CORRECTIVE/PREVENTIVE ACTION

14. Description of proposed action (use continuation page if necessary) Check the dimensions of the cold feet pads before their assembly on the cold mass and correct if necessary. This problem already appeared in other cold masses produced in your site.

| APPROVAL OF NON CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 15 | Project Engineer: Marta Bajko | Date: 4/2/03 |

| APPROVAL OF CRITICAL NON CONFORMITIES |  |  |
| --- | --- | --- |
| 16 | Project Management: | Date: |

| CLOSURE OF THE NON CONFORMITY |  |  |
| --- | --- | --- |
| Planned actions have been completed and corrective/preventive actions have been initiated |  |  |
|  | For non critical non conformities Quality Manager or Project Engineer | For critical non conformities Project Engineer |
| 17 | Name: F. Terzi Date: 02/04/03 | Name: Date: |

10. Description of non conformity (continuation page)

Please find here below the results of geometrical test regarding feet positioning. (ref. to Excel data file ITP20-HCMBB-2012.xls).

F. Terzi