|  | NON CONFORMITY REPORT | Nr. NC CERN 70 (CERN NC id. num.) |
| --- | --- | --- |
| Ansaldo Superconduttori S.p.A. |  |  |

| CUSTOMER CERN |  | JOB 0209EM |  | PROJECT NAME LHC 30 cold masses |  |  | Nr. RNC 020904A (ANSALDO NC id. num.) Date: 04/08/2002 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFICATION Cold mass M006 |  |  |  |  |  |  |  MAJOR  minor |  |
| REFERENCE DOCUMENTS 620RM08234/I; 620RM08213/D |  |  |  |  |  |  |  |  |
| NON CONFORMITY DESCRIPTION |  |  |  |  |  |  |  |  |
| Geometrical measurements performed on M006, installed on welding press under a pressure of 250 bar, before longitudinal welding, showed a twist of ± 7 mrad between cold bore tubes (nom. value ± 3 mrad max.) This non-conformity comes from problems created by the malfunctioning of the welding press, with consequent misalignment between upper and intermediate beams, occurred on week nr.30 during the first pressure cycle on M006. Note: the beams have been re-aligned on week 31 and an electro-valve has been substituted on week 36 |  |  |  |  |  |  |  |  |
| RESOLUTION |  |  |  |  |  |  |  |  |
| We propose to remove the upper shrinking cylinder, dismount the upper half yoke, remove the collared coils, re-align the lower half yoke and the lower shrinking cylinder, re-assemble all the components. |  |  |  |  |  |  |  |  |
| INTERNAL APPROVAL |  |  |  |  |  |  |  |  |
|  | PREPARED |  | CHECKED |  |  |  |  | APPROVED |
| Dept. | PRC |  | TEC |  |  |  |  | TEC |
| Name | B. Caserza |  | G. Drago |  |  |  |  | F. Terzi |
| Signature |  |  |  |  |  |  |  |  |
| Date | 03/09/2002 |  | 04/09/2002 |  |  |  |  | 04/09/2002 |

| CORRECTIVE ACTIONS |  |  |  |
| --- | --- | --- | --- |
| CUSTOMER APPROVAL |  |  |  |
|  ACCEPTED by F.Savary on the 2nd of September 2002 |  |  NOT ACCEPTED |  |
| REMARKS |  |  |  |
| CORRECTIVE ACTION EXECUTED |  |  | REPORTS ISSUED |
| Date 08/09/2002 | Signature F. Terzi |  |  |
| REMARKS |  |  |  |

MOD NC-CERN_2